Networking 3 Project ZNTW H3201 (Draft)

| Semester: | 2025 Sem A |
| --- | --- |
| Lecturer | Dr. James Egan |
| Learning Outcomes Covered |  | The following learning outcomes will be assessed: On successful completion of this module the learner should be able to:. LO1; Explain how to mitigate threats and enhance network security using access control lists and security best practices.. LO2; Install, configure and evaluate appropriate technologies and protocols for LAN interconnection and VPNs.. LO3; Implement techniques to provide address scalability and secure remote access for WANs.. |
| Weighting: | - 75% of the overall mark - 40% for project PacketTracer File, you will demo this to your lecturer in early December. The 40% also includes attendance and interaction with the project process. - Project due date is Friday the 5th of December 2025 at 23:59pm. Please upload both a Project PacketTracer and Project Report to Blackboard. Each group member should upload a PacketTracer and a Report. - 35% for project defence on Monday the 8th of December 2025 at 10am in C303 and C305. - The project defence is used to assess how much work has been contributed by each team member, the lecturer reserves the right to alter the 40% project score for a student based on the project defence. |
| Instructions | - Implement in groups of 2 or 3. - Complete the project one step at a time; before you begin one step ensure that you have completed all previous steps. - You should form your own groups. - If you cannot form your own group then please email me and I will form a group for you. - A 35% project defence will follow the week after the completion of this project. - Your group should implement each step in Packet Tracer. You will submit a PacketTracer and also a report that contains a) the VLSM addressing scheme that you used for your project b) Screenshots of testing for each stage. E.g. pings, show vlan brief, show ip route, show ip nat translations, show access-lists, show ip crypto sa - This is the print version of the project brief, please check Blackboard for digital copy as other helpful items may be added on this digital copy as the semester progresses. |

Scenario: Integration of Network Infrastructure at the National Children’s Hospital (NCH)

The National Children’s Hospital (NCH) is nearing completion and will become the largest healthcare facility for children in Ireland. It is being developed by integrating several existing paediatric services currently spread across Dublin — including Our Lady’s Children’s Hospital, Crumlin, Temple Street Children’s University Hospital, and the National Children's Hospital at Tallaght University Hospital — into one state-of-the-art facility.

The hospital will operate across several departments, satellite clinics, and support facilities. The integration of IT infrastructure across all current and future locations has been approved for the 2026 period.

The approximate staff numbers and functions at each of the existing and future sites are detailed in Table 1 below:

Table 1: Staff and Department Breakdown by Location

| Location | Total Staff (Approx) | Departmental Breakdown |
| --- | --- | --- |
| St. James's Campus (Main Hospital) | 1,200 | 15 Senior Management, 25 HR, 300 Admin, 30 IT, 500 Medical & Nursing Staff, 100 Allied Health Professionals, 50 Research Staff, 20 Device Management Addresses, 160 Other |
| Crumlin (Legacy Site) | 610 | 10 Senior Management, 15 HR, 200 Admin, 20 IT, 300 Medical & Nursing, 50 Other, 15 Device Management Addresses |
| Temple Street (Legacy Site) | 550 | 10 Senior Management, 10 HR, 150 Admin, 15 IT, 300 Medical & Nursing, 50 Other, 15 Device Management Addresses |
| Tallaght (Legacy Site) | 400 | 8 Senior Management, 10 HR, 120 Admin, 10 IT, 200 Medical & Nursing, 30 Other, 10 Device Management Addresses |
| Connolly Hospital Outpatient Clinic (Future) | 150 | 5 Senior Management, 5 HR, 30 Admin, 5 IT, 80 Medical & Nursing, 15 Other, 10 Device Management Addresses |
| Digital Health & Research Hub (Future) | 120 | 5 Senior Management, 10 Researchers, 10 Admin, 5 IT, 5 Device Management Addresses, 85 Other |

Email from CEO to IT Manager

Subject: NCH Integrated Network Planning – Immediate Action Required

Dear IT Manager,

As we move toward the final phases of transitioning to the new National Children’s Hospital, I am requesting that a full plan for the integrated network infrastructure be drawn up and tested in the coming weeks.

We need a future-proof, scalable, and secure network that connects all departments, clinics, and support units across our current and future locations. Please ensure this network plan includes high availability, security, and redundancy features.

I look forward to your proposal.

Kind regards,
 CEO, National Children’s Hospital

Email from IT Manager to Network Technician Team

Subject: Network Infrastructure Plan for National Children’s Hospital

Dear IT Team,

We have received a directive from the CEO to begin planning the network infrastructure for the new National Children’s Hospital (NCH), including all current and future locations.

I have outlined the basic specifications and it is up to you to....breakdown by location in the attached documentation. Your task is to develop a comprehensive implementation plan and create a prototype of the network using simulation software (e.g., Cisco Packet Tracer, GNS3, or similar).

Please include considerations for:

- Secure communication between all facilities

- Device management

- Remote access

- Integration of medical IoT devices

Let me know if you require any additional information.

Best regards,
 IT Manager
 National Children’s Hospital

Notes

1) You do not have to implement every user in PacketTracer. If there are 10 IT Staff in a Hospital you only have to put two PCs (the 1st and last). If there are 50 Nurses again you only have to implement two PCs (the 1st and last). E.g. You have to do the IP address planning for 50 PCs but only implement 2 PCs.

2)  Students should download and use the latest version of PacketTracer for this project from the Cisco Netacad site.

3) Students should form their own groups and email this information to your lecturer by Friday the 19th of September at 2pm. If you are not in a group by Friday at 2pm, I will form a group for you

4) MS Teams will be used to view the Collaboration between group members. Each group will be assigned a group name.

5) Do not put any passwords on any devices initially. Set a hostname on each device, we want to access the devices as quickly as possible for our configurations and testing.

Project Steps

Step 1

The Local Area Network (LAN) configuration in each site should separate users of different types. The network has to be optimized so that the LANs operate as efficiently as possible and also users of different type can be separated for security reasons. Please implement VLANs and use VLAN Trunking protocol (VTP) to ease the configuration.

Step 2

Design a private IPv4 addressing scheme for the network. Also select an appropriate routing protocol.

Note: Use private address for your internal IP addresses e.g. 10.0.0.0, 172.16.0.0, 192.168.0.0. Use a tool such as https://vsmcalc.vercel.app/ to help you form an addressing plan. You must select a private IP address to use that is unique to you. Put your name next to your selected range on this online spreadsheet. Select Here

Note: Make a copy of your VLAN and IP address ranges from the VLSM Calculation and put this information into a document, you will upload this at the end. I recommend using the 2901 router, do not use the Router called Router-PT. You can use the 2900 series Switch.

Step 3

Any guests who enter NCHs premises should have access to the network using the Wireless LAN called Guests. Configure a VLAN for this WLAN, as the premises are large a Wireless Controller (WC) and Access Points (APs) will be needed at each site. Note: Each group member should implement wireless at one of their sites.

Step 4

To provide an Internet service to their staff and guests the public IP addresses assigned for SETU are in the 149.153.1.0/27 range and the 193.1.184.0/23 range. For connecting ISPs together you can use 87.44.50.0/24.

Network Address Translation/Port Address Translation (NAT/PAT) is mandatory at each location.

NCHs email server and web server will also be located at the Temple Street site and these servers will use a public IP address.

Step 5

All of NCH sites are not going to be directly connected together but will use the Internet and a WAN technology which provides a VPN between the sites. The traffic between the St. James's Campus and Temple Street and at least one other set of branches should be encrypted (at least two other sets of branches for groups of size 3).

Step 6 #

There are a number of different users on the network such as IT staff, Medical, Accounting, HR, etc. We wish to restrict certain network access rights for different users.

- The IT staff should be the only staff members who can access network devices through SSH.

- Block “Admin” staff from accessing everything except for the servers and the Internet.

- HR should be allowed to access all parts of the network but all other aspects of the network should not be able to access HR staff.

- Allow the Internet to access the servers in Temple Street for email and Internet traffic only. Block access to all other parts of the network.

How to start

- Form a group, once formed email James the group members.

- Select 2 sites that you are going to implement.

- Select an IP address range from the Network ranges spreadsheet in step 2. Each person will use this range of addresses for their own 2 site.

- Use the VLSM calculator to create network ranges for each VLAN in your network. E.g. IT staff, Medical, Accounting, HR,

- Show James the plan

- Start to configure a topology.

- Upload all of your progress into the group project onto the Teams communications channel. This will enable James to see who is working and who is not working.